# Survey for CCF analysis:

All answers applied to specific CCCG

## 1: Input similarity-Subfactor Score

For this survey, the best option at the moment is to take the average of the 5 questions. note that each of the question 1a thru 1e have 5 possible values 0, .25, .5, .75, 1.

Index = (what + where + format + how + when)/5

### Information/input similarity Question 1a (What)

Information in this case refers to the What of information. What is the information or variable of interest (pressure, temperature, etc.) and is that shared by the members of the CCCG?

Question: Considering the elements of the CCCG, to what extent do the elements rely on the same “What” information?

- ☐ Not at all

- ☐ To a small extent

- ☐ To a moderate extent

- ☐ To a great extent

- ☐ Fully and systematically

### Information/input similarity Question 1b (Where)

Information in this case refers to the Where of information. Where does the information come from (pressure sensor, human messenger, external system, etc.)  and is that shared by the members of the CCCG?

Question: Considering the elements of the CCCG, to what extent do the elements rely on the same “where” information?

- ☐ Not at all

- ☐ To a small extent

- ☐ To a moderate extent

- ☐ To a great extent

- ☐ Fully and systematically

### Information/input similarity Question 1c (Format)

Information in this case refers to the format of information. What is the format of the information or data and is that the same format employed for all members of the CCCG?

Question: Considering the elements of the CCCG, to what extent do the elements rely on the same “format” information?

- ☐ Not at all

- ☐ To a small extent

- ☐ To a moderate extent

- ☐ To a great extent

- ☐ Fully and systematically

### Information/input similarity Question 1d (How)

Information in this case refers to the means of information. How is the information transferred (wireless, fiberoptic, wired, etc.) and is that the same means employed for all members of the CCCG?

Question: Considering the elements of the CCCG, to what extent do the elements rely on the same “how” information?

- ☐ Not at all

- ☐ To a small extent

- ☐ To a moderate extent

- ☐ To a great extent

- ☐ Fully and systematically

### Information/input similarity Question 1e (When)

Information in this case refers to the when of information. When is the information required (i.e. timing of information) and is that the same timing employed for all members of the CCCG?

Question: Considering the elements of the CCCG, to what extent do the elements rely on the same “when” information?

- ☐ Not at all

- ☐ To a small extent

- ☐ To a moderate extent

- ☐ To a great extent

- ☐ Fully and systematically

## 2: Understanding – Subfactor Score

Use the table below:

| Subfactor Score | Criterion (UT=U1+U2+U3+U4) |
| --- | --- |
| A | U=4 |
| B | U=3 |
| C | U=2 |
| D | U=1 |
| E | U=0 |

### Question 2a: Understanding-Novelty

Question: Considering the elements of the CCCG, to what extent do the elements rely on the novel, principles, configurations, or  information?

- ☐ Not Novel. The CCCG elements contain well-understood concepts, reused technology, or similar configurations to previously implemented systems. (Score U1=0)

- ☐ Novel. The CCCG can be described as first-of-a-kind, software with limited operational experience, or contains concepts that are not fully understood. (Score U1=1)

### Question 2b: Understanding - Complexity

For the purposes of this work, a system is considered complex if it performs two or more distinct functions.

Question: Do the members of the CCCG perform only a single dedicated function/action?

- ☐ Yes (Score U2=0)

- ☐ No. (Score U2=1)

### Question 2c: Understanding – Misfit

A question of off-the-shelf vs purpose-built software. Does the elements of the CCCG mostly comprise of off-the-shelf systems, pre-existing functions, or established functions

Question: Do the members of the CCCG ?

- ☐ High misfit: The system has high “misfit” if the if the majority of the system’s functionality is accomplished through off-the-shelf systems, pre-existing functions, or established functions. (Score U3=0)

- ☐ Low misfit: The system has low “misfit” if there are zero to minimal off-the-shelf systems, pre-existing functions, or established functions. (mostly purpose-built functions). (Score U3=1)

### Question 2d: Understanding – Experience

Experience is credited when there is 10 or more years of operational data for the CCCG. For new systems, credit will be given if the CCCG design employs software that has 10 years of operating experience (e.g., using an off-the-shelf product that has 10 years of experience in another system)

Question: Indicate the operational experience of the CCCG

- ☐ operational experience is more than 10 years. (Score U4=0)

- ☐ operational experience is less than 10 years. (Score U4=1)

## 3: AFA – Subfactor Score

| Analysis And Feedback | No Evidence of Awareness (Simple redundancy of 2) L1 | Evidence of General Awareness (2+ or complex redundancy) L2 | Evidence of Specific Awareness (use of diversity) L3 |
| --- | --- | --- | --- |
| No-An, No-F | A | B | C |
| An, No-F |  |  |  |
| An+, No-F |  |  |  |
| No-An, F |  |  |  |
| An, F | B | C | D |
| An+, F | B | C | D |
| No-An, F+ |  |  |  |
| An, F+ | B | C | D |
| An+, F+ | B | D | E |
| No-An = No Analysis; An = Analysis; An+ = Detailed Analysis; No-F = No Feedback; F = Feedback: F+ = Detailed Feedback |  |  |  |

### Question 3a: AFA – Analysis

Analysis considers the assessment that was performed for the CCF of the CCCG and may include a hazard or probabilistic risk assessment (PRA) performed system development.

Question: Indicate the level of risk analysis that was performed for the CCCG

- ☐ Level 1 The CCCG has not been identified and no CCF analysis has been carried out for the specific CCCG (Score = No-An)

- ☐ Level 2 The CCCG is identified as a potential hazard. Little consideration is made to how a CCF within a CCCG might influence the system (Score = An).

- ☐ Level 3 The CCCG is identified and analyzed for its threat to system performance. CCF is considered. Hazard and consequences are tracked as part of an FTA or other formal PRA tools (An+).

### Question 3b: AFA – Feedback

Feedback refers to the information and recommendations provided to the system designers/developers regarding CCFs of the CCCG.

Question: Indicate the level of feedback that was performed for the CCCG

- ☐ Level 1 No feedback concerning the CCCG was provided to the design team (Score = No-F)

- ☐ Level 2 Feedback was provided to the design team concerning the specific CCCG (Score = F).

- ☐ Level 3 Detailed feedback was provided to the design team. There is evidence that feedback led to actionable recommendations that are document trackable (Score = F+).

### Question 3c: AFA – Awareness

Awareness: refers to the designer’s knowledge and experience with CCFs and application of that knowledge in design. Credit for a designer’s awareness is made by considering evidence of design’s defenses against CCF.

Question: Indicate the level of awareness of CCF as evident in the design with respect to the CCCG.

- ☐ Level 1 There is no evidence of awareness for CCF of the CCCG. No dedicated attempt to prevent software CCF was included in the design (i.e., duplication of software in redundant trains). The CCCG has low levels or no built-in redundancy (Score = Level 1)

- ☐ Level 2 There is evidence of general CCF knowledge as demonstrated by the existence of redundant configurations within the design. No diversity is used to support design. (Score = Level 2).

- ☐ Level 3 There is evidence of awareness of software-based CCFs. Diverse software configurations are used. The analyst may also reason that there are other advanced methods beyond diversity that merit a score beyond Level 2. (Score = Level 3)

## 4: Human-Machine Interface – Subfactor Score

Score: both columns are assessed. The limiting or score is selected.

| Score | Control Of Operator/User Interactions | Maintenance Interactions |
| --- | --- | --- |
| A | No procedures or guidance over operator/user interaction | No supervisory checking |
| B | [Written procedures AND Normal interaction] OR [No Procedures AND minimal interaction] | Work checked by a supervisor |
| C | [Checklist AND Normal interaction] OR [Written Procedures AND minimal interaction] | Post maintenance proof testing |
| D | [Checklist with evidence that it is used AND Normal interaction] OR [Checklist AND minimal interaction] OR [Procedures with evidence of compliance AND minimal interaction] | Work is tested and checked |
| E | [Checklists with evidence of use AND minimal interaction] may also consider if the interaction is alarmed to ensure correctness in the same way as for maintenance | Maintenance activities have associated acceptance tests to ensure correctness |
| Note: both columns are assessed. If scores differ, the more limited (i.e., pessimistic) score is selected |  |  |

|  | Frequency |  |
| --- | --- | --- |
| Control | Regular (normal interaction) | Minimal (infrequent interaction) |
| No procedures/ guidance for interactions | A | B |
| Written procedures/ guidance. | B | C |
| Procedures AND [Checklist OR Review/Verification] | C | D |
| Procedures AND [Checklist OR Review/Verification] AND [evidence of use and compliance OR Use of automatic alerts] | D | E |
| *Evidence can be considered if documentation of maintenance reports, effort, etc. or in the case of non-existent systems, evidence in the form of planned documentation processes document handling plans etc. |  |  |

### Question 4a: Human-Machine Interface – Interaction frequency

Question: Indicate the level of interaction the user/operator/staff has with the CCCG

- ☐ Interactions are low or infrequent (Score = minimal).

- ☐ Interactions are regular, normal, scheduled, or routine (Score = Normal).

### Question 4b: Human-Machine Interface – Operator/User (op-Control)

Question: Indicate how user/operator interactions are controlled for the CCCG

- ☐ No procedures: There are no written procedures or guidance to interact with the CCCG.

- ☐ Procedures: There are written procedures to control the operation.

- ☐ Checklists: Checklists to compliment procedures.

### Question 4c: Human-Machine Interface – Maintenance (m-control)

Question: Indicate how maintenance activities are controlled for the CCCG.

- ☐ No guidance or controls

- ☐ Work checked by a supervisor.

- ☐ Post maintenance testing.

- ☐ Work checked and tested.

- ☐ All maintenance activities have specific acceptance tests.

## 5: Safety Culture & Training – Subfactor Score

Score: both columns are assessed. The limiting or score is selected.

| Score | Safety Culture | Education |
| --- | --- | --- |
| A | Casual | On-the-job |
| B | Casual | General Education |
| C | Safety Oriented AND (regular OR infrequent safety training) | General Education |
| D | Safety Oriented AND (regular safety training) | Specialized (may include simulations) |
| E | Safety Oriented+ | Specialized (may include simulations) AND multiple years of experience with the system |
| Note: both columns are assessed. If scores differ, the more limited (i.e., pessimistic) score is selected |  |  |

### Question 5a: Safety Culture & Training – (Training)

Question: What level of education has the staff received regarding the components and software of the CCCG?

- ☐ On-the-job: No formalized education concerning any aspect of the CCCG.

- ☐ General: Basic general education concerning the CCCG.

- ☐ Specialized: Detailed training, and education related to the specific components and software of the CCCG. May include simulator training, when applicable. Also, years of experience may be considered.

### Question 5b: Safety Culture & Training – (Safety Culture)

Question: What level of education has the staff received regarding the components and software of the CCCG?

- ☐ Casual: Staff may or may not have safety in mind. No safety training.

- ☐ Safety Oriented: Staff have safety in mind. Periodic safety training related to working with the components/software of the CCCG.

- ☐  Safety Oriented+: Staff have safety in mind, there is a clearly defined organizational safety culture, safety policies, regular safety training.

## 6: Access Control – Subfactor Score

Score based directly from answer selected

Question: What level of access control is in place for the components and software of the CCCG?

- ☐ A: No control, open access networks, unsecured physical locations.

- ☐ B: Secured physical locations, private networks, general institutional access, multiple unrelated software systems found in single physical location.

- ☐ C: Secured physical locations, private networks, limited institutional access (e.g., authorized personnel only, and passwords).

- ☐ D: Secured physical locations, private networks, limited access to authorized and trained personnel only. Close supervision is employed. The area where software is found is limited to software of similar purposes (i.e., multiple software programs on the same machine but all related to similar purpose) multiple systems may be present in the same area.

- ☐ E: Secured physical locations, private networks, extremely limited access, trained personnel only operating under close supervision, specialized machines (i.e., no other software present), only a single-purpose system is present in the area.

## 7: Testing – Subfactor Score

Score based directly from answer selected

Question: What level of testing is planned or has been implemented for the CCCG?

- ☐ A: No testing of the system, specifically the CCCG.

- ☐ B: Individual unit testing (single examples for each software type within CCCG). An example unit has been tested.

- ☐ C: Detailed testing is performed on an example system (i.e., CCCG). Testing includes verification and compliance testing to ensure the CCCG meets all required criteria as a unit.

- ☐ D: Commissioning tests performed on the specific CCCG to be employed. Detailed integration testing of the CCCG, in addition to stress testing.

- ☐ E: In addition to C&D levels, a long-term test is conducted for the CCCG. The test is performed in parallel with existing system for approximately for a specified duration (e.g.,1 year.)